[image: image3.png]50° |

60°

70°

80°

90°

100°





Product specification
	Model: YQW032A/YTW032A


	File No：xx-xxxxxx

	Version: V1.0


	Date
	Version
	Change history
	Author
	Approve

	2016.12.07
	V1.0
	
	
	

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


Best for T6cm+ huge light box lighting
YQW032A/YTW032A

[image: image2.png]Light up your brand

‘:; SunsignLED



1. Features
Constant Current Technology (CCT)，
The Aluminum plate for heat dissipation

Energy saving，Environment friendly

Long life span

2. Applications

Commercial standing lighting

Light box lighting

Logo lighting

Line/shape lighting 

3. Dimensions
[image: image1.jpg]18mm





Remark：The pitch of the light-bar is variable.
4. Environmental and Application Conditions

	Ambient temperature range (TA on free air)
	-25…..+55℃

	Operating (case) temperature range (TC)
	-25…..+55℃

	Storage temperature range
	-25…..+65℃

	IP rating
	IP62


5. Electrical parameters

	Supply voltage range
	12 VDC

	Supply Current range (per module)
	420mA/1.25A

	Power range (per module)
	5W/15W

	ESD protection
	Contact 4KV, Air 8KV


6. Optical parameters (module base)

	Part No.
	Color Range
	Flux Range
	Beam angle

	
	Min. CCT or wavelength
	Typ. CCT or wavelength
	Max. CCT or wavelength
	Min. Lumen
	Typ. Lumen
	

	YQW032A
	6000 K
	6500 K
	7000k
	450 lm
	460 lm
	  170º

	YTW032A
	6000 K
	6500 K
	7000k
	1400lm
	1500lm
	170º


7. LED module light distribution graph

[image: image3.png]
8. Attention

Max. number of modules of one chain is 1pcs from power supplier to single end.

The given illuminance parameters ensure illuminance on the surface. Depending on the type of material used for channel letters or lighting boxed,the parameters can change.More brightness is obtained if distances between LED modules are reduced.

The LED module itself and all its components may not be mechanical stressed.

Please ensure that the power supply is adequate power to operate the total load. Only qualified personnel should be allowed to perform installations.

The design of the housing should be according to the IP standards in the application.

If surge protection structure not within power supplier, a lightening protector should be needed additionally.















